# ASONA DEFENDER 50 ACOUSTICAL CEILING

This section deals with the manufacture, supply and installation of Asona ceiling systems,

GUIDANCE NOTE

Accessible ceilings comprising individual tiles and panels supported on an exposed grid system, Check the manufacturer’s data sheets and modify this section accordingly.

## 1.	GENERAL

GUIDANCE NOTE

Masterspec does not recommend the use of SCOPE or SCOPE OF WORK clauses.  However there are some forms of contract where a general description of what is covered by a particular work section can help guide those using the specification.  If you prefer to include such a general description, add it directly underneath the work section title, not as a numbered clause.

Related work

### 1.1	RELATED SECTIONS

Refer to ~ for ~

GUIDANCE NOTE

Include cross references to other sections where these contain related work.

Documents

### 1.2	DOCUMENTS

Documents referred to in this section are:

NZS 1170	Structural design actions, Earthquake actions – New Zealand

ISO 5660.1 	Cone calorimeter test reaction to fire tests for heat release, smoke production and mass loss rate.

NZBC C/AS1	Fire Safety

AS/NZS 2785	Suspended ceilings - Design and installation

AS 2946	Suspended ceilings, recessed luminaries and air diffusers - Interface requirements for physical compatibility

NZS 4219	Seismic resistance of engineering systems in buildings

ISO 354	Test method for sound absorption by reverberant room method

ISO 11654	Weighted sound absorption ratings

ASTM C423	Test method for sound absorption and sound absorption coefficients by the reverberation room method

ASTM E1414	Standard test method for airborne sound attenuation between rooms sharing a common ceiling plenum (two room method)

ASTM C 635	Standard specification for the manufacture, performance and testing of metal suspension systems for acoustical tile and lay-in panel ceilings

ASTM C636	Standard practice for installation of metal ceiling suspension systems for acoustical tile and lay-in panels

ASTM E413-87	Test method for airborne sound transmission STC

Documents listed above and cited in the clauses that follow are part of this specification.  However, this specification takes precedence in the event of it being at variance with the cited document.

GUIDANCE NOTE

Delete from the DOCUMENTS clause any document not cited.  List any additional cited documents.

RELATED DOCUMENTS

Refer to the following related documents when preparing this section:

ISO 11654	Ratings standard for sound absprption

BRANZ Bulletin 369: Fitting tolerances

### 1.3	MANUFACTURER’S DOCUMENTS

Manufacturer’s and supplier’s documents relating to work in this section are:

Asona Defender 50 data sheets

Asona T24 suspension system

Copies of the above literature are available online at www.asona.co.nz .

For lead times and pricing contact Asona Ltd, Unit 14, 7 Cain Road, Penrose, Auckland.

Email:	info@asona.co.nz

Telephone:	09 525 6575

GUIDANCE NOTE

It is important to ensure that all personnel on site have access to accurate, up to date technical information on the many products, materials and equipment used on a project.  In most cases individual products are not used in isolation, but form part of a building process.  Also a particular manufacturer’s and/or supplier’s requirements for handling, storage, preparation, installation, finishing and protection of their product can vary from what might be considered the norm.  Access to technical information can help overcome this potential problem.

Requirements

### 1.4	SAMPLE SECTION

Allow to erect a sample section of the suspended ceiling system offered.  Subject to confirmation in writing, the sample section may form part of the completed installation.

Refer to 4. SELECTIONS for location.

GUIDANCE NOTE

Amend or expand this clause as appropriate to the project, including requirements for incorporating other elements - luminaires, diffusers, surrounding surfaces - into the sample section.

### 1.5	INSTALLATION

Minimum installation to AS/NZS 2785 and ASTM C636.  Installation to be only by the manufacturer’s accredited installer, using the manufacturer’s technical services.  Accredited installers to be members of the Association of Wall and Ceiling Industries (AWCI) of New Zealand. Provide evidence of experience, listing completed projects of similar size and complexity.

Installation to NZS 4219; with related building services installations complying specifically with clauses 2.22, 2.25 and 2.29.

### 1.6	CLEANING

Clean using soft bristled brush, vacuum or a damp cloth or as noted in product data sheet.

### 1.7	SPARES

Provide spare matching ceiling elements in the quantities specified below.  Deliver into a dry store at the site or elsewhere as directed and at agreed times.  Refer to 4. SELECTIONS for quantity.

GUIDANCE NOTE

Ask for packs of tiles.

### 1.8	SUPPLY WARRANTY

Supply manufacturers 1 year warranty against manufacturing defects and 5 year NZBC B2 durability for tiles and 15 years for suspension components.

1.9	NO SUBSTITUTIONS

Substitutions are not permitted to any specified Asona system, or associated components and products.

Performance

GUIDANCE NOTE

Use these clauses if specifying by performance.  Even when specifying by brand it may still be necessary to include some reference to performance.

### 1.10	LOADING CODE REQUIREMENT

Comply with the requirements of:

-	NZS 4203, clause 4.12, Requirements for parts; or

-	NZS 1170, section 8, Requirements for parts and components

### 1.11	CERTIFICATION

Provide:

-	certification of compliance with:

NZS 1170, section 8, Requirements for parts and components

-	certificates and other evidence that the system offered complies with the standards of performance specified; and

-	a Producer Statement on completion.

### 1.12	ACOUSTIC REQUIREMENTS

Verify material sound absorption performance. Submit a copy of a full scale sound absorption test report to ISO 354 E200 as tested by Auckland Uni Services or other independent testing authority.  Refer to 4. SELECTIONS for acoustic performance requirements.

### 1.13	REACTION TO FIRE PROPERTIES

Reaction to fire properties of the ceiling system must meet the acceptable solutions to all risk groups C/AS1 to C/AS7. Risk groups: SH,SM,SI,CA,WB,WS,VP

ASONA PRODUCT FIRE GROUP NUMBER RATINGS

1S = Highest rating      Ref: ISO 5660-1 Cone Calorimeter test

| ASONA PRODUCTS | TYPE OF MATERIAL | GROUP NUMBER |
| --- | --- | --- |
| Defender | Glass fibre & metal ceiling panels | 1S |

GUIDANCE NOTE

Consult your fire engineer for advice.

### 1.14	ENVIRONMENTAL REQUIREMENTS

Design the ceiling system for use over its expected life without deterioration within the required temperature and humidity range.  Refer to 4. SELECTIONS for details.

GUIDANCE NOTE

If this clause is not used and the SITE CONDITIONS clause cannot be complied with, then allowance needs to be made for a tile and sheet linings that will not sag over the construction period.

### 1.15	LIGHT REFLECTANCE

To ASTM C1477.  Refer to 4 SELECTIONS for reflectance and colour.

GUIDANCE NOTE

Data from selected product technical data sheet

## 2.	PRODUCTS

GUIDANCE NOTE

Select the proprietary system to suit the brief.  Use the PRODUCTS clauses to get the desired finish and required fire and acoustic characteristics.  Use the EXECUTION clauses required for the selected system.

Materials – Accessible grid and tile ceiling systems

### 2.1	GRID SUSPENSION SYSTEM - STEEL

Roll formed hot dip galvanized steel elements to ASTM C635 intermediate or heavy duty for carrying ceiling panels, light fixtures and air distribution elements and to:

-	NZS 1170, section 8, Requirements for parts and components

GUIDANCE NOTE

Confirm the compatibility of recessed lighting, air conditioning unit and ventilation outlets to the ceiling grid.

### 2.2	PERFORATED STEEL COMPOSITE PANELS

Asona Defender 50™ abuse resistant composite acoustical panel from perforated steel facer and Sonatex impact interlayer laminated to high density bio soluble resin bonded glass fibre absorber. Made in NZ

GUIDANCE NOTE

Specify abuse resistant perforated steel composite panels for areas requiring class A high sound absorption for control of reverberation, high impact resistance, non combustibility.  Best for prisons, sports halls, gyms, corridors etc ,

## 3.	EXECUTION

Conditions

### 3.1	CO-ORDINATE SERVICES

Co-ordinate and co-operate with electrical and mechanical work to avoid conflict between suspension members and luminaires, diffusers, pipework and ducting.  Confirm the provision of extra hangers and fixings.

Ensure co-operation with work in and above the ceiling, including the marking of specific ceiling tiles below major access points to above-ceiling services.  Colour coded markings to follow the standards laid down by mechanical and electrical services.

### 3.2	SITE CONDITIONS

Do not begin installation until the building is closed in, fully glazed, the roof watertight, and mechanical and electrical duct work above the ceiling completed.

GUIDANCE NOTE

If fast track construction is used, this may not be possible and high humidity resistant non sag soft fibre tiles must be selected accordingly.

### 3.3	COMPATIBILITY

To AS 2946 for interface requirements for physical compatibility.

### 3.4	RESPONSIBILITY

Ensure that conditions are suitable for the ceiling installation.  Arrange for the programming of the work to suit required practice.

Application

### 3.5	INSTALL

Install the system to AS/NZS 2785 minimum standards and the ceiling manufacturer's requirements.

### 3.6	ACCESSIBILITY

Provide access to the ceiling system and the in ceiling and above ceiling services so that maintenance and removal of any part can be carried out without damage to the ceiling system or panels.

GUIDANCE NOTE

As full access in all areas is clearly impractical, any special requirements for clear access will need to be defined; either by description here, or on the drawings.

### 3.7	PENETRATIONS

Accommodate recessed light fittings, air conditioning outlets and other electrical and/or mechanical services that are fixed to or pass through the ceiling system. Provide independent support for these as necessary. Fittings > 2kg are not to be supported by the acoustical ceiling panels.

### 3.8	PROTECT EXISTING WORK

Protect adjacent existing work from damage during the installation.  Where selected provide service panels finished with a temporary protective film.  Remove on completion of the ceiling installation.

GUIDANCE NOTE

Protective film is recommended especially for white panels to reduce damage and wastage caused by other workers

Completion

### 3.9	REPLACE

Replace or repair damaged or marked elements.  Recycle material, consult manufacturer for recoating procedure.

### 3.10	LEAVE

Leave work to the standard required by following procedures.

### 3.11	REMOVE

Remove debris, unused elements and elements from the site.

### 3.12	CLEAN

Clean soiled or marked units.

## 4.	SELECTIONS

Requirements

### 4.1	SAMPLE SECTION

Location:	~

Size: 	~ metres x ~ metres

Performance

GUIDANCE NOTE

Use these performance clauses only where system is being specified by performance; otherwise delete all performance clauses and specify under system

### ACOUSTIC REQUIREMENTS

Practical sound absorption properties to ISO 354 E200, ratings to ISO 11654

| Frequency | 125 Hz | 250 Hz | 500 Hz | 1000 Hz | 2000 Hz | 4000 Hz |
| --- | --- | --- | --- | --- | --- | --- |
| P | 0.45 | 0.80 | 0.85 | 0.90 | 0.90 | 0.80 |

Sound absorption class:		A

Weighted sound absorption coefficient W:	0.90

NRC to ASTM C423-99:		0.85

### 4.3	ENVIRONMENTAL REQUIREMENTS

Range:	10 - 45°C

Relative humidity:	95 % maximum

GUIDANCE NOTE

If this clause is not used and the SITE CONDITIONS clause cannot be complied with, then allowance needs to be made for a tile that will not sag over the construction period.

### 4.4	REFLECTANCE

Reflectance:	LR1 per ASTM 1477, >75 %

For (colour):	White

4.5	FIRE

Group Number: 1S (IS0 5660.1 Cone calorimeter test)

GUIDANCE NOTE

Data from selected product technical data sheet

### 4.6	SCHEDULE

| Area | Product reference | Finish |
| --- | --- | --- |
| ~ | Asona Defender 50 | Powder coat white |

GUIDANCE NOTE

Identify areas and note product type, finish and acoustical ratings required.

4.7	CEILING PANEL

Brand/product:	Asona Defender 50™

Manufacturer:	Asona Ltd, Unit 14, 7 Cain Road, Penrose, Auckland

Tel: 09 525 6575, Fax: 09 525 6579, email: info@asona.co.nz

Material type:	Perforated steel facer, Sonatex impact interlayer, dual layer high density glass fibre absorber.

Thickness:	50 mm

Size: 	(600x1200 mm) (600x1800 mm) (600x2400 mm)

Edge:	Square

Finish: 	Powder coat

Colour: 	Pacific White (other)

Security:	(Pop Rivet Panels to T38 grid + edge trim. (CLW 095 hold down clips @ 300 mm centres)

### 4.8	TEMPORARY PROTECT FILM

Required for panels located adjacent to:

Sprinkler heads

Light fittings including down lights

Speakers

HVAC grilles

Other ceiling penetrations

GUIDANCE NOTE

Delete this clause if ceiling panel protection is not required

### 4.9	SUSPENSION GRID

Type:	Asona T38 wall faced heavy duty grid

Material: 	Galvanised steel

Finish/colour: 	White~

Module: 	to suit panel.

Hanger:	(Direct fix clip) (2.5 mm hanger wire) (Solid hanger)

GUIDANCE NOTE

Type:

T24 	24 mm faced exposed grid, white paint coated, economical

Hanger:	Standard 2.5 mm hanger wire fixed to grid

Option of  direct fix CL070 clip, face of ceiling 80 mm from soffit.

### 4.10	PERIMETER TRIM

Type:	WA1922 19 x 22 mm profile

Material:	Pre-painted galvanised steel body

Finish/colour:	White

GUIDANCE NOTE

Type:

WA2022	20 mm x 22 mm wall angle, white paint coated galvanized steel body

WA2040	20 mm x 40 mm wall angle, white paint coated galvanized steel body

WAS1010	Shadowline wall angle, 10 mm x 10 mm negative, white coated steel body

Spares

### 4.11	SPARES

Panels:	~

TOVC: 	low	GUIDANCE NOTE

Ask for packs of tiles.