# ASONA FABWALL, FABRIC COVERED WALL PANELS

This section deals with the manufacture, supply and installation of Asona ceiling & associated acoustical wall elements offered by Asona Limited.

GUIDANCE NOTE

Check the manufacturer’s data sheets and modify this section accordingly.

## 1.	GENERAL

GUIDANCE NOTE

Masterspec does not recommend the use of SCOPE or SCOPE OF WORK clauses.  However there are some forms of contract where a general description of what is covered by a particular work section can help guide those using the specification.  If you prefer to include such a general description, add it directly underneath the work section title, not as a numbered clause.

Related work

### 1.1	RELATED SECTIONS

Refer to ~ for ~

GUIDANCE NOTE

Include cross references to other sections where these contain related work.

Documents

### 1.2	DOCUMENTS

Documents referred to in this section are:

NZBC C/AS1-7	Spread of fire

4.15 Concealed spaces

4.17 Interior surface finishes, floor coverings and suspended flexible fabrics

ISO 5660.1 	Cone calorimeter test reaction to fire tests for heat release, smoke production and mass loss rate.

ISO 354	Acoustics - Measurement of sound absorption in a reverberation room

ISO 11654	Acoustics - Sound absorbers for use in buildings - Rating of sound absorption

ASTM C423	Test method for sound absorption and sound absorption coefficients by the reverberation room method

Documents listed above and cited in the clauses that follow are part of this specification.  However, this specification takes precedence in the event of it being at variance with the cited document.

GUIDANCE NOTE

Delete from the DOCUMENTS clause any document not cited.  List any additional cited documents.

RELATED DOCUMENTS

Refer to the following related documents when preparing this section:

ISO 11654	Ratings standard for sound absprption

BRANZ Bulletin 369: Fitting tolerances

### 1.3	MANUFACTURER’S DOCUMENTS

Manufacturer’s and supplier’s documents relating to work in this section are:

Asona Fabwall data sheet

Copies of the above literature are available online at www.asona.co.nz .

For lead times and pricing contact Asona Ltd, info@asona.co.nz

Telephone:	09 525 6575

Mobile:	021 505 740

GUIDANCE NOTE

It is important to ensure that all personnel on site have access to accurate, up to date technical information on the many products, materials and equipment used on a project.  In most cases individual products are not used in isolation, but form part of a building process.  Also a particular manufacturer’s and/or supplier’s requirements for handling, storage, preparation, installation, finishing and protection of their product can vary from what might be considered the norm.  Access to technical information can help overcome this potential problem.

Requirements

### 1.4	SAMPLE SECTION

Allow to erect a sample section of the acoustic wall panels offered.  Subject to confirmation in writing, the sample section may form part of the completed installation.

Refer to 4. SELECTIONS for location.

GUIDANCE NOTE

Amend or expand this clause as appropriate to the project, including requirements for incorporating other elements - luminaires, diffusers, surrounding surfaces - into the sample section.

### 1.5	INSTALLATION

Installation to be only by the manufacturer’s accredited installer, using the manufacturer’s technical services. Provide evidence of experience, listing completed projects of similar size and complexity.

### 1.6	CLEANING

Clean using soft bristled brush or vacuum, consult fabric manufacturers recommendations as noted in product data sheet.

### 1.7	SPARES

Provide spare matching ceiling elements in the quantities specified below.  Deliver into a dry store at the site or elsewhere as directed and at agreed times.  Refer to 4. SELECTIONS for quantity.

GUIDANCE NOTE

Ask for no. panels

### 1.8	SUPPLY WARRANTY

Supply manufacturers 5 year warranty against manufacturing defects and 5 year NZBC B2 durability for panels.

1.9	NO SUBSTITUTIONS

Substitutions are not permitted to any specified Asona system, or associated components and products.

Performance

GUIDANCE NOTE

Use these clauses if specifying by performance.  Even when specifying by brand it may still be necessary to include some reference to performance.

### 1.10	CERTIFICATION

Provide:

-	a producer statement on completion.

### 1.11	ACOUSTIC REQUIREMENTS

Verify material sound absorption performance. Submit a copy of a full scale sound absorption test report to ISO 354 E200 as tested by Auckland University Services or other independent testing authority.  Refer to 4. SELECTIONS for acoustic performance requirements.

### 1.12	REACTION TO FIRE PROPERTIES

Reaction to fire properties of the ceiling system must meet the acceptable solutions to all risk groups C/AS1 to C/AS7. Risk groups: SH,SM,SI,CA,WB,WS,VP

ASONA PRODUCT FIRE GROUP NUMBER RATINGS

ISO 5660-1 Cone Calorimeter test

| ASONA PRODUCTS | GLASS FIBRE BOARDS | GROUP NUMBER |
| --- | --- | --- |
| Fabwall | 475 gsm Coated glass fiber fabric wrapped | 1 S |
| Fabwall | 250 gsm Polyester fabric wrapped panel | 2 |
| Fabwall | 450 gsm Polyester fabric wrapped panel | 2 |
| Fabwall | 300 gsm Wool fabric wrapped panel | 2 |
| Fabwall | 150 gsm Linen fabric wrapped panel | 2 |

GUIDANCE NOTE

Key criteria are spread of flame and smoke developed indices. The surface of some fabrics  may have an initial ignitability and heat evolved index greater than 0, consult your fire engineer for advice and discuss with the manufacturers before specifying.

### 1.13	ENVIRONMENTAL REQUIREMENTS

Design the fabric panel system for use over its expected life without deterioration within the required temperature and humidity range.  Refer to 4. SELECTIONS for details.

GUIDANCE NOTE

If this clause is not used and the SITE CONDITIONS clause cannot be complied with, then allowance needs to be made for a fabric  lining that will not sag over the construction period.

### 1.14	LIGHT REFLECTANCE

To ASTM C1477.  Refer to 4 SELECTIONS for reflectance and colour.

GUIDANCE NOTE

Data from selected product technical data sheet

## 2.	PRODUCTS

GUIDANCE NOTE

Select the proprietary system to suit the brief.  Use the PRODUCTS clauses to get the desired finish and required fire and acoustic characteristics.  Use the EXECUTION clauses required for the selected system.

2.1	FABRIC COVERED CEILING AND WALL PANELS

Asona Fabwall™ factory manufactured fabric wrapped acoustic panels from selected decorative fabric membrane stretched over (25,40,50,75,100,150mm) high density glass fibre core with 2 ply Sonatex composite impact interlayer, steel or resin reinforced edges.

Manufacturer: Asona Ltd Tel: 09 525 6575, Fax: 09 525 6579, email: info@asona.co.nz

GUIDANCE NOTE

Specify fabric acoustic ceiling and wall finish for applications that require high sound absorption and decorative fabric finish. Best for meeting rooms, quiet rooms, lobbies, conference centres, hotel, cinema, recording studios etc. Consult Asona for panel options.

## 3.	EXECUTION

Conditions

### 3.1	CO-ORDINATE SERVICES

Co-ordinate and co-operate with electrical and mechanical work to avoid conflict between penetrations, suspension members and luminaires, pipe work and ducting.  Confirm the provision of extra fixings.

### 3.2	SITE CONDITIONS

Do not begin installation until the building is closed in, dry and mechanical and electrical duct work completed and interior is clean and free of dust and dirt.

GUIDANCE NOTE

Fabric panels are a finishing trade and installation must be at end of contract.

### 3.3	COMPATIBILITY

To AS 2946 for interface requirements for physical compatibility.

### 3.4	RESPONSIBILITY

Ensure that conditions are suitable for the fabric panel installation.  Arrange for the programming of the work to suit required practice.

Application

### 3.5	INSTALLATION

Install the system to manufacturer's requirements Mount panels to structure using factory supplied Z bar, L brackets on glue fix to structure

### 3.6	PENETRATIONS

Accommodate recessed light fittings, and other electrical and/or mechanical services that are fixed to or pass through the fabric panels. Provide independent support for these as necessary. Such fittings are not to be supported by the fabric panels.

### 3.7	PROTECT EXISTING WORK

Protect adjacent existing work from damage during the installation.  Where selected provide service panels finished with a temporary protective film.  Remove on completion of the ceiling installation.

GUIDANCE NOTE

Completion

### 3.8	REPLACE

Replace or repair damaged or marked elements.

### 3.9	LEAVE

Leave work to the standard required by following procedures.

### 3.10	REMOVE

Remove debris, unused elements and elements from the site.

### 3.11	CLEAN

Clean soiled or marked units.

## 4.	SELECTIONS

Requirements

### 4.1	SAMPLE SECTION

Location:	~

Size: 	~ metres x ~ metres

Performance

### ACOUSTIC REQUIREMENTS

NRC to ASTM C423-99:		Fabwall 25 – NRC 0.85

Fabwall 40 – NRC 0.95

Fabwall 50 – NRC 1.00

Fabwall 75 – NRC 1.00

Fabwall 100- NRC 1.00

Fabwall 150- NRC 1.00

Fabwall 150 Bass – NRC 0.10

GUIDANCE NOTE

Select panel type to meet acoustical absorption required.

### 4.3	ENVIRONMENTAL REQUIREMENTS

Range:	10 - 45°C

Relative humidity:	 95 % maximum

4.4	FIRE

Group Number:	1S, 2S, 2, 3 (IS0 5660.1 Cone calorimeter test)

As per fabric selection

GUIDANCE NOTE

If this clause is not used and the SITE CONDITIONS clause cannot be complied with, then allowance needs to be made for a tile that will not sag over the construction period.

### 4.5	SCHEDULE

| Area | Product reference | Finish |
| --- | --- | --- |
| ~ | Asona Fabwall | - 475 Glass Fibre - 150 GSM Linen - 250 GSM Polyester - 300 GSM Wool - 450 GSM Polyester - Other |

GUIDANCE NOTE

Identify areas and note product type and fabric  finish  required.

PANELS

### 4.6	FABRIC PANEL

Brand/product:	Asona Fabwall™

Manufacturer:	Asona Ltd, 7 Cain Rd, Penrose, Auckland

Tel: 09 525 6575, Fax: 09 525 6579, email: info@asona.co.nz

Material:	Bio soluble glass fibre core with 4 ply Sonatex glass mat impact interlayer, steel frame and wrapped in Charade decorative acoustically transparent fabric as selected

Thickness:	(25),(40),(50),(75),(100),(150) mm

Size: 	600 x 600mm)(600 x 1200)(600 x 1800)(600 x 2400)(1200 x 1200)(1200 x 1800)(1200 x 2400) (other)

Finish: 	(475 Glass Fibre Fabric, 150 GSM Linen, 250 GSM Polyester, 300 GSM Wool, 450 GSM Polyester, Other) (MT PCV coated glass fibre scrim) (other)

Colour: 	as selected

GUIDANCE NOTE

A wide range of decorative acoustically transparent fabrics are available, contact Asona Ltd for options. Thicker panels provide high levels of sound absorption especially at mid to low frequencies. Thick panels recommended for large volume spaces

Spares

### 4.7	SPARES

Panels:	~

GUIDANCE NOTE

Ask for packs of tiles.